In this problem, four energy producers participate in a day-ahead merit-order electricity market, including three conventional ones and a renewable one. These producers submit hourly bids that specify a delivery quantity of electricity and an offer price for each hour of the next day. The market operator uses these bids to determine the market-clearing price and dispatch schedule. If a bid is accepted, the producer is obligated to deliver the specified quantity of electricity at the clearing price. Otherwise, the producer earns zero profit for that hour.

However, renewable generation is deeply uncertain, as it depends on weather conditions that cannot be predicted. If the producer under-delivers relative to its bid (due to overestimation of production), or over-delivers (due to underestimation), penalties or imbalance costs would apply. These costs are imposed by the system operator to maintain grid stability and to ensure supply-demand balance in real time. Meanwhile, the clearing price is unknown to the producer in advance, so they need to compete in this market.

The renewable energy producer’s goal is to maximize its expected profit by strategically selecting the quantity of electricity to bid for each hour of the next day, accounting for the uncertainty in both renewable generation and clearing prices.

Market structure and timing:

1. The day-ahead market is cleared once per day (e.g., at noon) for 24 hourly intervals of the following day.

2. Each producer submits a bid $(b_t, p_t)$ for each hour $t=1, …, 24$, where:

2.1. $b_t$: the quantity of energy (in MWh) the producer is willing to supply.

2.2. $p_t$: the minimum acceptable price per MWh.

3. The market operator computes the market-clearing price $c_t$ for each hour.

4. If $p_t \leq c_t$, the bid is accepted, and the producer is committed to delivering $b_t$ MWh in hour $t$.

5. The total energy demand in the market is $D_t$, which follows a normal distribution with constant mean $\mu_D$ and variance ${\sigma_D}^2$.

Penalty structure:

1. If actual generation $P_t < b_t$, the shortfall $b_t - P_t$ incurs a penalty of $q_u$ per MWh.

2. If actual generation $P_t \geq b_t$, the surplus $P_t - b_t$ incurs a curtailment penalty of $q_o$ per MWh.

Conventional producers:

1. For each conventional producer $i$, its bid $(b_{it}, p_{it})$ for each hour $t$ follows normal distributions with constant means and variances.

Please formalize this problem description as a model. Please also specify key components, such as the state variables, initial state, uncertainties and transition functions.